*****************************************************************************

*                      (c) Freescale  Inc. 2004 All rights reserved

*

* File Name     : debug.c

*

* PURPOSE: Diagnostics output through SCI

*

*

* DESCRIPTION: Implementation of the diagnostics output - enabled when

*              ETH_DEBUG keyword is defined (-DETH_DEBUG)

*              _SCI_BASE must be set to SCI0 or SCI1

*              (see compiler command line arguments)

*

* Version : 2.1

* Date    : 02/04/04

*

*****************************************************************************/

//#include <stdio.h>

#include "ne64debug.h"

#include "MC9S12NE64.h"

#include "eeprom.h"

/*************shell fun used sci0*******************/

extern tU08 hard_addr[6];

extern tU08 prot_addr[4];

extern tU08 netw_mask[4];

extern tU08 dfgw_addr[4];

extern tU08 brcs_addr[4];

extern tU08 DEMO_RMTHOST_IP[4];

extern tU16 DEMO_LOCAL_PORT;

extern tU16 DEMO_RMTHOST_PRT;

tU08 TCPIP_MODE=0;

tU32 SCI_BAUD;

tU16 SCI_BASE;

tU08 SCI_MODE;

tU08 SCI_BAUDtemp;

tU08 prot_addrf,netw_maskf,dfgw_addrf,DEMO_RMTHOST_IPf,

DEMO_LOCAL_PORTf,DEMO_RMTHOST_PRTf,mac_addf,

sci_modef,set_baudf,tcp_modef;

/******************************/

#ifdef ETH_DEBUG

#define _BASE 0x0000  /**< Base on register map */

#define ECLK 25000000 /**< this is BUSCLK */

#define reg(x)  *((volatile tU08 *)(_BASE+x))

#define regw(x)  *((volatile tU16 *)(_BASE+x))

//#define	_SCI_BASE   SCI0

//#ifndef _SCI_BASE

//#error _SCI_BASE not defined, use SCI0 or SCI1

//#endif

#define SCI0    0x00c8

#define SCI1    0x00d0

#define SCIBD  regw(SCI_BASE+0x00)

#define SCICR1 reg(SCI_BASE+0x02)

#define SCICR2 reg(SCI_BASE+0x03)

#define SCISR1 reg(SCI_BASE+0x04)

#define SCIDRL reg(SCI_BASE+0x07)

//===================================================

void InitDebug(void)

{

#define BAUD_RATE    9600

#define BAUD_DIV     ECLK/16/BAUD_RATE

SCI_BASE  = SCI0;

SCIBD= BAUD_DIV;

SCICR1= 0;

//SCICR2= SCI1CR2_TE_MASK | SCI1CR2_RE_MASK | SCI1CR2_RWU_MASK ;

SCICR2 = 0x2C;

}

//===================================================

/* send string via SCI */

//===================================================

void Debugt(tS08 * s)

{

while (*s != 0)

{

while (!(SCISR1 & 0x80));

SCIDRL=*s;

s++;

}

}

//===================================================

/* send string via SCI for protocol display*/

//===================================================

void Debugs(tU08  s)

{

while (!(SCISR1 & 0x80));

SCIDRL=s;

}

//===================================================

/* convert 8bit number to 2 hexadecimal ascii characters */

//===================================================

tS08 cvt[16]={ '0','1','2','3','4','5','6','7','8','9',

'A','B','C','D','E','F' };

//===================================================

void Ctoh (tS08 * s, tU08 c)

{

*s= cvt[c / 16];

s++;

*s= cvt[c % 16];

}

//===================================================

/* send 16bit number in hexa via SCI */

//===================================================

void Debugi(tU16 i)

{

tS08 s[6];

s[0]=' ';

Ctoh(s+1,(tU08)(i / 256));

Ctoh(s+3,i % 256);

s[5]=0;

Debugt(s);

}

//===================================================

/* send 8bit number in hexa via SCI */

//===================================================

void Debugc(tU08 c)

{

tS08 s[4];

s[0]=' ';

Ctoh(s+1,c);

s[3]=0;

Debugt(s);

}

//===================================================

/* send newline and return characters via SCI */

//===================================================

void Debugnl(void)

{

Debugt("\r\n");

}

/*********************shell function**************************/

//===================================================

/* shell write data to IPADDRESS/NET MASK/ROUTE RECEIVE buffer */

//===================================================

tU08 shell_recedata(tU08 *p)

{

tU08 dataok=0,s;

tU08 i,j=0;

tU08 bufftemp[3];

for(i=0;i<4;)

{

while(!(SCISR1 & 0x20));

s = SCIDRL;

if((s !='.')&&(s !=0x0D)){

if(('0'<= s)&&(s <='9')){

if(j==3){

Debugs(0x07);

// dataok=1;

// break;

} else{

Debugs(s);

s = s&0x0f;

bufftemp[j]=s;

j++;

}

} else if(s==0x08)

{

if(j!=0){

Debugs(0x08);

j--;

} else {

Debugs(0x07);

}

} else{

Debugs(0x07);

//dataok = 1;

//break;

}

// bufftemp[j]=s;

// j++;

} else

{

if((s=='.')&&(i!=3)&&(j!=0))

{

Debugt(".");

if(j==1)p[i]=bufftemp[0];

if(j==2)p[i]=bufftemp[1]+bufftemp[0]*10;

if((j==3)&&(bufftemp[0]<3))p[i]=bufftemp[2]+bufftemp[1]*10+bufftemp[0]*100;

else if((j==3)&&(bufftemp[0]>3))dataok=1;

i++;

j=0;

}else if((s==0x0D)&&(i==3)&&(j!=0))

{

if(j==1)p[i]=bufftemp[0];

if(j==2)p[i]=bufftemp[1]+bufftemp[0]*10;

if((j==3)&&(bufftemp[0]<3))p[i]=bufftemp[2]+bufftemp[1]*10+bufftemp[0]*100;

else if((j==3)&&(bufftemp[0]>3))dataok=1;

i++;

j=0;

} else if((s==0x0D)&&(i==0)&&(j==0)){

dataok=1;

return(dataok);

} else {

Debugs(0x07);

// dataok = 1;

}

}

}

return(dataok);

}

//===================================================

/* shell MAC ADDRESS STR TO NUM funciton select via SCI */

//===================================================

tU08 strTOnum(tU08 s)

{

if(('0'<= s)&&(s <='9')){

s = s&0x0f;

}

else if(('A'<= s)&&(s <='F')){

s=s&0x0f;

s+=9;

}

else if(('a'<= s)&&(s <='f')){

s=s&0x0f;

s+=9;

}

else if(s==0x08){

//Debugs(0x08);

return(128);

}

else return(255);

return(s);

}

//===================================================

/* shell  configration SCI BAUD_RATE process via SCI */

//===================================================

void shell_scibaud(void)

{

tU08 i,ss;

set_baudf=0;

for(i=0;i<1;)

{

while(!(SCISR1 & 0x20));

ss = SCIDRL;

if(ss==0x0D) {

set_baudf=1;

break;

}

SCI_BAUDtemp= strTOnum(ss);

if(SCI_BAUDtemp!=255)

{

Debugs(ss);

i++;

}

else{

Debugs(0x07);

}

}

}

//===================================================

/* SET SCI BAUD_RATE process via SCI */

//===================================================

void set_scibaud(void) {

switch(SCI_BAUDtemp)

{

case 1:

SCI_BAUD=300;

break;

case 2:

SCI_BAUD=600;

break;

case 3:

SCI_BAUD=1200;

break;

case 4:

SCI_BAUD=2400;

break;

case 5:

SCI_BAUD=4800;

break;

case 6:

SCI_BAUD=9600;

break;

case 7:

SCI_BAUD=19200;

break;

case 8:

SCI_BAUD=38400;

break;

case 9:

SCI_BAUD=57600;

break;

case 10:

SCI_BAUD=115200;

break;

case 11:

SCI_BAUD=230400;

break;

case 12:

SCI_BAUD=460800;

break;

default:

break;

}

}

//===================================================

/* shell  configration remote port and local port process via SCI */

//===================================================

tU08 shell_cofigport(tU16 *port)

{

tU08 s,i;

tU08 bufftemp[5];

tU08 dataok=0;

for(i=0;i<5;i++){

bufftemp[i]=0;

}

for(i=0;i<6;)

{

while(!(SCISR1 & 0x20));

s = SCIDRL;

if(s !=0x0D){

if(('0'<= s)&&(s <='9')&&(i<5)){

Debugs(s);

s = s&0x0f;

bufftemp[i]=s;

i++;

} else if(s==0x08)

{

if(i!=0){

Debugs(0x08);

i--;

} else {

Debugs(0x07);

}

} else{

Debugs(0x07);

}

} else

{

if(i==4){

*port=bufftemp[0]*1000;

*port+=bufftemp[1]*100;

*port+=bufftemp[2]*10;

*port+=bufftemp[3];

if((*port)<1024) {

Debugs(0x07);

} else return(0);

} else if(i==5)

{

*port=bufftemp[0]*10000;

*port+=bufftemp[1]*1000;

*port+=bufftemp[2]*100;

*port+=bufftemp[3]*10;

*port+=bufftemp[4];

if((bufftemp[0]>6)&&((*port)<1024)) {

//i=5;

Debugs(0x07);

} else return(0);

} else if(i==0){

dataok=1;

return (dataok);

}else Debugs(0x07);

}

}

}

//===================================================

/* shell funciton configration process via SCI */

//===================================================

void shell_func(void)

{

tU08 i,ss;

//tU08 hard_addrf,prot_addrf;

// tU08 netw_maskf,dfgw_addrf;

tU08 hard_addrtemp[12];

//help ifo

Debugnl();

Debugt("            HELP INFORMATION,\r\n");

Debugt("The Follow Is The Info Should Be Configed\r\n");

Debugt("MAC ADDRESS By      0x00-0xFF\r\n");

Debugt("IP  ADDRESS By      0-255\r\n");

Debugt("NET MASK By         0-255\r\n");

Debugt("DEFAULT ROUTE By    0-255\r\n");

Debugt("DEMO_RMTHOST_IP By  0-255\r\n");

Debugt("TCPIP_MODE By       0,1,2\r\n");

Debugt("REMOTE HOST PORT By 1024-50000\r\n");

Debugt("LOCAL HOST PORT By  1024-50000\r\n");

Debugt("SCI_MODE SELECT By  0,1\r\n");

Debugt("SCI BAUD_RATE       1-C\r\n");

/*  Debugt("You Can Choose The Figure To Confige The Parameter Separately\r\n");

Debugt("Or Input 'S' To Confige The Patameter Step by Step\r\n");

while(!(SCISR1 & 0x20));

s = SCIDRL;

s = s&0x0f;*/

//MAC ADDRESS configration

// Debugnl();

Debugnl();

Debugt("MAC ADDRESS (000000000000):");

mac_addf=0;

for(i=0;i<12;)

{

while(!(SCISR1 & 0x20));

ss = SCIDRL;

if((ss==0x0D)&&(i==0)) {

mac_addf=1;

break;

}

hard_addrtemp[i]= strTOnum(ss);

if(hard_addrtemp[i]!=255)

{

if(hard_addrtemp[i]==128){

if((i!=0)&&(i%2)){

Debugs(0x08);

i--;

} else{

Debugs(0x07);

}

}else {

Debugs(ss);

i++;

if((i!=0)&&(i!=12)){

if(!(i%2))

Debugt("-");

}

}

} else{

Debugs(0x07);

//hard_addrf = 1;

}

}

// if(hard_addrf==0){

for(i=0;i<6;i++){

hard_addr[i]=hard_addrtemp[i*2]*16+ hard_addrtemp[i*2+1];

}

// }

//IP address configration

Debugnl();

Debugt("IP ADDRESS       (0.0.0.0):");

prot_addrf=shell_recedata(prot_addr);

//NET MASK configration

Debugnl();

Debugt("NET MASK         (0.0.0.0):");

netw_maskf=shell_recedata(netw_mask);

//DEFAULT ROUTE configration

Debugnl();

Debugt("DEFAULT ROUTE    (0.0.0.0):");

dfgw_addrf=shell_recedata(dfgw_addr);

//DEMO_RMTHOST_IP configration

Debugnl();

Debugt("DEMO_RMTHOST_IP  (0.0.0.0):");

DEMO_RMTHOST_IPf=shell_recedata(DEMO_RMTHOST_IP);

//TCPIP_MODE SELECK

Debugnl();

Debugt("TCPIP_MODE SELECT:\r\n");

Debugt("     0:TCPIP_MODE_UDP\r\n");

Debugt("     1:TCPIP_MODE_TCP_CLIENT\r\n");

Debugt("     2:TCPIP_MODE_TCP_SERVER\r\n");

tcp_modef=0;

for(i=0;i<1;)

{

while(!(SCISR1 & 0x20));

ss = SCIDRL;

if(ss==0x0D) {

tcp_modef=1;

break;

}

if(('0'<=ss)&&(ss<='2'))

{

Debugnl();

Debugs(ss);

ss = ss&0x0f;

TCPIP_MODE =ss;

i++;

} else{

Debugs(0x07);

}

}

//Remote Port

Debugnl();

Debugt("REMOTE HOST PORT (00000) By 1024-50000:\r\n");

DEMO_RMTHOST_PRTf=shell_cofigport(&DEMO_RMTHOST_PRT);

//LOCAL Port

Debugnl();

Debugt("LOCAL HOST PORT (00000) By  1024-50000:\r\n");

DEMO_LOCAL_PORTf=shell_cofigport(&DEMO_LOCAL_PORT);

// SCI_MODE SELECT configration

Debugnl();

Debugt("SCI_MODE SELECK:\r\n");

Debugt("     0:SCI0_MODE\r\n");

Debugt("     1:SCI1_MODE\r\n");

sci_modef=0;

for(i=0;i<1;)

{

while(!(SCISR1 & 0x20));

ss = SCIDRL;

if(ss==0x0D) {

sci_modef=1;

break;

}

if(('0'<=ss)&&(ss<='1'))

{

Debugnl();

Debugs(ss);

ss = ss&0x0f;

SCI_MODE =ss;

i++;

} else{

Debugs(0x07);

}

}

// SCI BAUD_RATE configration

Debugnl();

Debugt("SCI BAUD_RATE By\r\n");

Debugt("  1.300;  2.600;  3.1200; 4.2400;  5.4800;  6.9600; \r\n");

Debugt("  7.19200;8.38400;9.57600;A.115200;B.230400;C.460800.\r\n");

shell_scibaud();

set_scibaud();

}

//===================================================

/* configration info write to eeprom for save*/

//===================================================

void shell_eepromwrite(void){

tU16 eeprom_buf[3];

tU08 i;

Debugnl();

Debugnl();

Debugt("System Is Going to Write Info to EEPROM");

//IP address write to eeprom

if(!prot_addrf)

{

for(i=0;i<2;i++)

{

Erase_eeprom(20+i);

eeprom_buf[i] = prot_addr[i*2+1]*256 + prot_addr[i*2];

Write_eeprom(20+i,eeprom_buf[i]);

}

}

//NET MASK write to eeprom

if(!netw_maskf)

{

for(i=0;i<2;i++)

{

Erase_eeprom(22+i);

eeprom_buf[i] = netw_mask[i*2+1]*256 + netw_mask[i*2];

Write_eeprom(22+i,eeprom_buf[i]);

}

}

//DEFAULT ROUTE write to eeprom

if(!dfgw_addrf)

{

for(i=0;i<2;i++)

{

Erase_eeprom(24+i);

eeprom_buf[i] = dfgw_addr[i*2+1]*256 + dfgw_addr[i*2];

Write_eeprom(24+i,eeprom_buf[i]);

}

}

//DEMO_RMTHOST_IP configration

if(!DEMO_RMTHOST_IPf)

{

for(i=0;i<2;i++)

{

Erase_eeprom(26+i);

eeprom_buf[i] = DEMO_RMTHOST_IP[i*2+1]*256 + DEMO_RMTHOST_IP[i*2];

Write_eeprom(26+i,eeprom_buf[i]);

}

}

//MAC ADDRESS configration

if(!mac_addf)

{

for(i=0;i<3;i++)

{

Erase_eeprom(28+i);

eeprom_buf[i] = hard_addr[i*2+1]*256 + hard_addr[i*2];

Write_eeprom(28+i,eeprom_buf[i]);

}

}

//TCPIP_MODE SELECK

if(!tcp_modef)

{

Erase_eeprom(31);

eeprom_buf[0]=(tU16)(TCPIP_MODE);

Write_eeprom(31,eeprom_buf[0]);

}

//Remote Port

if(!DEMO_RMTHOST_PRTf)

{

Erase_eeprom(32);

eeprom_buf[0]= DEMO_RMTHOST_PRT;

Write_eeprom(32,eeprom_buf[0]);

}

//LOCAL Port

if(!DEMO_LOCAL_PORTf)

{

Erase_eeprom(33);

eeprom_buf[0]= DEMO_LOCAL_PORT;

Write_eeprom(33,eeprom_buf[0]);

}

// SCI BAUD_RATE configration

if(!set_baudf)

{

Erase_eeprom(34);

eeprom_buf[0]=(tU16)(SCI_BAUDtemp);

Write_eeprom(34,eeprom_buf[0]);

}

// SCI_MODE SELECT configration

if(!sci_modef)

{

Erase_eeprom(35);

eeprom_buf[0]=(tU16)(SCI_MODE);

Write_eeprom(35,eeprom_buf[0]);

}

Debugnl();

Debugt("Write Process OK!\r\n");

}

//===================================================

/* read configration info form eeprom */

//===================================================

void shell_eepromread(void)

{

tU16 eeprom_buf[3];

tU08 i;

//IP address write to eeprom

for(i=0;i<2;i++){

eeprom_buf[i] = Read_eeprom(20+i);

prot_addr[i*2+1] =(tU08)(eeprom_buf[i]/256);

prot_addr[i*2] = eeprom_buf[i]%256;

}

//NET MASK write to eeprom

for(i=0;i<2;i++){

eeprom_buf[i] = Read_eeprom(22+i);

netw_mask[i*2+1] =(tU08)(eeprom_buf[i]/256);

netw_mask[i*2] = eeprom_buf[i]%256;

}

//DEFAULT ROUTE write to eeprom

for(i=0;i<2;i++){

eeprom_buf[i] = Read_eeprom(24+i);

dfgw_addr[i*2+1] =(tU08)(eeprom_buf[i]/256);

dfgw_addr[i*2] = eeprom_buf[i]%256;

}

//DEMO_RMTHOST_IP configration

for(i=0;i<2;i++){

eeprom_buf[i] = Read_eeprom(26+i);

DEMO_RMTHOST_IP[i*2+1] =(tU08)(eeprom_buf[i]/256);

DEMO_RMTHOST_IP[i*2] = eeprom_buf[i]%256;

}

//MAC ADDRESS configration

for(i=0;i<3;i++)

{

eeprom_buf[i] = Read_eeprom(28+i);

hard_addr[i*2+1] =(tU08)(eeprom_buf[i]/256);

hard_addr[i*2] = eeprom_buf[i]%256;

}

//TCPIP_MODE SELECK

eeprom_buf[0]=Read_eeprom(31);

TCPIP_MODE = (tU08)(eeprom_buf[0]);

//Remote Port

eeprom_buf[0]=Read_eeprom(32);

DEMO_RMTHOST_PRT = eeprom_buf[0] ;

//LOCAL Port

eeprom_buf[0]=Read_eeprom(33);

DEMO_LOCAL_PORT = eeprom_buf[0];

// SCI BAUD_RATE configration

eeprom_buf[0]=Read_eeprom(34);

SCI_BAUDtemp=(tU08)(eeprom_buf[0]);

set_scibaud();

// SCI_MODE SELECT configration

eeprom_buf[0]=Read_eeprom(35);

SCI_MODE=(tU08)(eeprom_buf[0]);

}

//===================================================

/* DISPLAY configration info  */

//===================================================

void shell_disp(void)

{

tU08 i;

if(prot_addrf==1) {

Debugnl();

Debugt("The Info Write to IP address Is Not Matching Standard\r\n");

prot_addrf=0;

}

if(netw_maskf==1) {

Debugnl();

Debugt("The Info Write to NET MASK Is Not Matching Standard\r\n");

netw_maskf=0;

}

if(dfgw_addrf==1) {

Debugnl();

Debugt("The Info Write to DEFAULT ROUTE Is Not Matching Standard\r\n");

dfgw_addrf=0;

}

if(DEMO_RMTHOST_IPf==1) {

Debugnl();

Debugt("The Info Write to DEMO_RMTHOST_IP Is Not Matching Standard\r\n");

DEMO_RMTHOST_IPf=0;

}

//MAC ADDRESS configration

Debugnl();

Debugt("MAC ADDRESS (000000000000):");

for(i=0;i<6;i++){

Debugc(hard_addr[i]);

}

//IP address configration

Debugnl();

Debugt("IP ADDRESS       (0.0.0.0):");

for(i=0;i<4;i++){

Debugc(prot_addr[i]);

}

//NET MASK configration

Debugnl();

Debugt("NET MASK         (0.0.0.0):");

for(i=0;i<4;i++){

Debugc(netw_mask[i]);

}

//DEFAULT ROUTE configration

Debugnl();

Debugt("DEFAULT ROUTE    (0.0.0.0):");

for(i=0;i<4;i++){

Debugc(dfgw_addr[i]);

}

//DEMO_RMTHOST_IP configration

Debugnl();

Debugnl();

Debugt("DEMO_RMTHOST_IP  (0.0.0.0):");

for(i=0;i<4;i++){

Debugc(DEMO_RMTHOST_IP[i]);

}

//TCPIP_MODE SELECK

Debugnl();

Debugt("TCPIP_MODE SELECK:");

if(TCPIP_MODE==0)Debugt("TCPIP_MODE_UDP");

if(TCPIP_MODE==1)Debugt("TCPIP_MODE_TCP_CLIENT");

if(TCPIP_MODE==2)Debugt("TCPIP_MODE_TCP_SERVER");

//Remote Port

Debugnl();

Debugt("REMOTE HOST PORT (00000):");

Debugi(DEMO_RMTHOST_PRT);

//LOCAL Port

Debugnl();

Debugt("LOCAL HOST PORT  (00000):");

Debugi(DEMO_LOCAL_PORT);

// SCI_MODE SELECT configration

Debugnl();

Debugnl();

Debugt("SCI_MODE SELECK:  ");

if(SCI_MODE==0)Debugt("SCI0_MODE");

if(SCI_MODE==1)Debugt("SCI1_MODE");

// SCI BAUD_RATE configration

Debugnl();

if(SCI_BAUDtemp==1)Debugt("SCI BAUD_RATE IS 300\r\n");

if(SCI_BAUDtemp==2)Debugt("SCI BAUD_RATE IS 600\r\n");

if(SCI_BAUDtemp==3)Debugt("SCI BAUD_RATE IS 1200\r\n");

if(SCI_BAUDtemp==4)Debugt("SCI BAUD_RATE IS 2400\r\n");

if(SCI_BAUDtemp==5)Debugt("SCI BAUD_RATE IS 4800\r\n");

if(SCI_BAUDtemp==6)Debugt("SCI BAUD_RATE IS 9600\r\n");

if(SCI_BAUDtemp==7)Debugt("SCI BAUD_RATE IS 19200\r\n");

if(SCI_BAUDtemp==8)Debugt("SCI BAUD_RATE IS 38400\r\n");

if(SCI_BAUDtemp==9)Debugt("SCI BAUD_RATE IS 57600\r\n");

if(SCI_BAUDtemp==10)Debugt("SCI BAUD_RATE IS 1152000\r\n");

if(SCI_BAUDtemp==11)Debugt("SCI BAUD_RATE IS 230400\r\n");

if(SCI_BAUDtemp==12)Debugt("SCI BAUD_RATE IS 460800\r\n");

Debugnl();

}

//===================================================

/* shell process init via SCI */

//===================================================

void shell_init(void)

{

UINT16 i,j;

UINT8 n;

tU16 dat;

Debugnl();

Debugnl();

Debugt("     =================================\r\n");

Debugt("     Welcome to System configure shell\r\n");

Debugt("     =================================\r\n");

Debugnl();

Debugt("I/O Port Test....              OK\r\n");

Debugt("System Clock Initialization... OK\r\n");

/*********for eeprom******************************/

// Write_eeprom(0x10,0x1278);

// dat=Read_eeprom(0x10);

//sprintf(buf,"dat=%d\n",dat);Debugt(buf);

if(dat==0x1278){

Debugt("EEPROM Initialization...       OK\r\n");

} else{

Debugt("EEPROM Initialization...    FAULT\r\n");

Debugt("SYSTEM Initialize To a Default Configration\r\n");

//shell_eepromread();      //eeprom is bad,so system should apply a default configration

//a function should insert here.

for(i=0;i<6;i++) {

hard_addr[i] = 0;

}

prot_addr[0]=192; prot_addr[1]=168; prot_addr[2]=0; prot_addr[3]=2;

netw_mask[0]=255; netw_mask[1]=255; netw_mask[2]=255; netw_mask[3]=0;

dfgw_addr[0]=192; dfgw_addr[1]=168; dfgw_addr[2]=0; dfgw_addr[3]=1;

brcs_addr[0]=192; brcs_addr[1]=168; brcs_addr[2]=0; brcs_addr[3]=255;

DEMO_RMTHOST_IP[0]=192; DEMO_RMTHOST_IP[1]=168; DEMO_RMTHOST_IP[2]=0; DEMO_RMTHOST_IP[3]=3;

SCI_MODE=0;

TCPIP_MODE = 0;

DEMO_RMTHOST_PRT=5001;

DEMO_LOCAL_PORT=5000;

SCI_BAUD=9600;

shell_disp();

/***********************************************/

Debugt("Press Any Key For The Begin of Configuration");

for(n=0;n<10;n++){

Debugt(".");

for(i=0;i<1000;i++){

for(j=0;j<8000;j++){}

if(SCISR1 & 0x20)

{

SCIDRL;

Debugnl();

shell_func();

shell_eepromwrite(); // function here  implement a fun

shell_eepromread();

shell_disp();                // to configration info into eeprom

return;

}

}

}

Debugnl();

Debugnl();

Debugt("SYSTEM Initialize To a Previous Configration\r\n");

shell_eepromread();

shell_disp();

// a fuction here should implement a fun to read info from eeprom

}

}

#endif