A Software Duplex UART for the 751/752                                  AN446

The following program contains routines that will allow an 8xC751 or 8xC752 to

implement a software UART that can send and receive serial data

simultaneously. Other published software UARTs only allow either transmit or

receive to occur at any one time. The demo application shown in the code

listing waits for data to be received, then echoes it and follows this with a

hexadecimal interpretation of the data plus a space. For instance, if the

program receives the character "$", it echoes back the string "$24".  The

reason for echoing these additional characters is to make it easy to force the

receiver buffer to fill up in order to test the handshaking. If the program

simply echoed what was received, it would likely never use more than the very

first receiver buffer location since it can normally transmit just as fast as

it can receive.

CHIP RESOURCES

The UART routines use about 400 bytes of code space and use the timer to

provide a constant time interrupt to synchronize both transmit and receive

operations. The hardware connections require four device pins to accomplish

serial I/O with RTS/CTS handshaking. Only two pins would be needed if

handshaking is not required. Three of the four pin functions may be assigned

to any port pin. The serial input pin must be assigned as one of the external

interrupt pins. Another two pins are used in the demo application to input a

selection of one of four baud rates (1200, 2400, 4800, or 9600).

LIMITATIONS

To obtain duplex operation, a fairly large portion of the chip's time is used.

The routines were tested up to 9600 baud running on a 16 MHz 87C751. When

serial input and output were both occurring at the same time, the routines

could not support continuous operation with no pauses between characters. At

4800 baud, full speed tight reception and transmission worked flawlessly. In

other words, 4800 baud should work with all applications, while 9600 baud may

not work with all applications.

THEORY OF OPERATION

There are three possible sequences of events when serial transmit and receive

may both be operating at once: transmit and receive begin simultaneously;

transmit is requested while the receiver is busy; and receive starts while the

transmitter is busy. The first 2 cases could be handled fairly simply with

only one interrupt for each bit time. In the first case, everything is already

in synch and only one timer and one interrupt per bit is needed to do both

operations. In the second case (transmit is requested while the receiver is

busy) the program could just wait for the next bit time to start transmitting.

Unfortunately, the third case presents a problem. If the program is already

transmitting, it cannot always wait for the next bit time to start sampling

the serial data if the application is not to lose bits. Also, the timer cannot

be adjusted to the incoming data since this would distort the duration of one

of the transmitted bits.

The method used here to deal with this problem is to always divide all bit

times into 4 sub-bit times. When transmission and/or reception is in progress,

the timer runs at 4X the bit rate for the selected baud rate. The variables

TxTime and RxTime are used to count sub-bit times for the transmitter and the

receiver, respectively. Both are initialized to a negative value and count up

to simplify testing for an active sub-bit time. The maximum baud rate that can

be supported is essentially determined by the maximum amount of time that it

might take the microcontroller to do all of the operations associated with

transmitting one bit and receiving one bit . This must be done within the time

between timer interrupts.

When both transmit and receive operations are scheduled for the same timer

interrupt, priority is given to the transmitter routine. The reason for this

is that a great deal of jitter can be tolerated in the timing of the received

bit sampling, but the transmitted data must "look" good to the outside world.

The actual bit times for transmit and receive are counted by the variables

TxCnt and RxCnt, respectively. When an active sub-bit time slice occurs, these

variables tell the transmit and receive routines what to do in the current

time slice. The value 11 hex indicates a start bit, 10 hex indicates a stop

bit, and the values 8 through F hex indicate a data bit. The values were

chosen to allow quick determination of the appropriate action by the code.

The routines provide for a small amount of data buffering for both the

transmitter and the receiver. As implemented here, the transmitter buffer is

only one byte deep, allowing one data byte to be held while another is being

transmitted. The receiver buffer is larger, allowing three bytes to be held

while a fourth is being received. If the receiver buffer fills up (indicated

by the flag RxFull), the application code must retrieve one byte before a

fourth one finishes, or data will be lost. If this happens, a flag will be set

(OverrunErr) to indicate that the receiver buffer has been overrun. There is

no similar flag for the transmitter, since the transmit request routine waits

for the transmitter buffer to be available (indicated by the TxFull flag)

before taking action. It is up to the application code to check this flag in

advance if it does not want to stall execution while waiting to transmit data.

As each routine finishes a whole data byte by completing the send or receive

of a stop bit, it checks to see if there is something still happening to

warrant having the time slice interrupt running. In the case of a received

stop, the transmit activity flag (TxOn) is examined. If it is not set, the

timer is turned off. The timer will be turned back on if an interrupt from a

serial start bit is received or the main code requests data to be transmitted.

In the case of a transmitted stop, both the receiver activity flag (RxOn) and

the transmit buffer flag (TxFull) are examined. If the receiver is active or

there is more data to transmit, leave the timer is left running.

All of the status flags are in the "Flags" register. Other status flags found

there are: RxAvail, which indicates that the receiver buffer contains

unprocessed data; and FramingErr which is set when the receiver routines find

an improper start or stop bit, usually caused by mismatched baud rates.

Flow control handshaking is provided by the RTS/CTS scheme. The transmit

routine looks at the incoming CTS line before beginning each start bit

transmission, and simply exits, waiting for the next time slice, if CTS is not

asserted. The receive routine checks the buffer status whenever a start bit

interrupt occurs and de-asserts the outgoing RTS line if the buffer already

contains two bytes (i.e., it will be full when the current byte finishes). If

the device at the other end of the communication line follows the same rules

(which may very well NOT be the case) the program should be able to

communicate without buffer overflows in either direction.

Baud rates in both the send and receive routines are determined by two things:

the timer interrupt rate; and the number of time slices per bit. The method of

calculating the timer value for various baud rates is discussed in the code

listing at the BaudRate routine. This discussion has centered on there being

four time slices per bit, but if the user wants, set either the transmitter or

the receiver can be set to run at a baud rate that is a multiple of the other

by adjusting the value of the constant TxBitLen or RxBitLen. The baud rate

would be calculated as indicated for the faster channel, and TxBitLen or

RxBitLen would be changed for the slower channel. For example, the transmitter

can be set to run at half of the receiver baud rate by setting TxBitLen to -8

+ 1.

The routines shown also make provision for changing the baud rate "on the

fly", although the application code given does not implement this feature. If

the application code changes the baud rate for some reason, the change will be

effected when the next data transmission or reception begins, if both the

transmitter and receiver were already idle. This prevents the timer value from

being changed in the middle of a data byte.

THE CODE

There are a number routines in the code that the user should be aware of:

Intr0     - Called (by interrupt) when a serial start bit is  received.

Timer0    - Called (by interrupt) for every sub-bit time slice.

RS232TX   - Called by Timer0 when the transmitter has business to conduct in

the current time slice.

RS232RX   - Called by Timer0 when the receiver has business to conduct in

the current time slice.

BaudRate  - Sets the baud rate variables BaudHigh and BaudLow based on the

accumulator value.

TxSend    - Called by the application code to request that a data byte be

transmitted. The data to be transmitted is in the accumulator.

GetRx     - Called by the application code to request return of a received

data byte from the buffer. Data is returned in the

accumulator. This routine should not be called unless the

receiver buffer has data available.

Reset     - Start of the initialization code to set up the UART.

MainLoop  - Start of the mainline code of the demo application.